# M-Core PICC Software Design

Version: 1.0
Date: 2026-01-14
Author: weizhichun (uix08649)
Scope: M-Core (S32G399A M7_0) Inter-Core Communication Implementation

## 1. Document Overview

This document describes the software architecture design of the M-Core Inter-Core Communication Middleware (PICC - Private Inter-Core Communication). The document adopts a top-down structure, progressing from the system global view to module details, enabling readers to quickly understand the overall architecture before diving into specific implementations.

### 1.1 Reading Guide

| Chapter | Content | Target Audience |
| --- | --- | --- |
| Chapter 2 | File List | All readers - Quick overview of involved files |
| Chapter 3 | System Global View | All readers - Quick overview of overall architecture |
| Chapter 4 | Layered Architecture | Developers - Understanding layer responsibilities |
| Chapter 5 | Core Process Diagrams | Developers - Understanding data flow and state transitions |
| Chapter 6 | Module Detailed Design | Developers - Implementation details reference |
| Chapter 7 | Task Scheduling Design | Developers - Understanding task execution mechanism |

## 2. File List

| File | Type | Description |
| --- | --- | --- |
| picc_api.h/c | Header/Source | PICC Unified External Interface |
| picc_protocol.h/c | Header/Source | Protocol Packet Definition & Processing |
| picc_service.h/c | Header/Source | Event/Method Service Layer |
| picc_stack.h/c | Header/Source | Message Stacking & CRC Processing |
| picc_link.h/c | Header/Source | Application Layer Connection Management |
| picc_heartbeat.h/c | Header/Source | Channel Layer Heartbeat Detection |
| picc_trace.h/c | Header/Source | Debug Tracing |
| picc_pwr_main.h/c | Header/Source | Power Business Interface Layer |
| picc_pwr_cnf.h/c | Header/Source | Power Module Configuration |
| Pwsm/pwsm.h/c | Header/Source | Power State Machine |
| Pwsm/pwsm_cnf.h/c | Header/Source | Power State Machine Configuration |
| main.c | Source | Main Program Entry |

## 3. System Global View

### 3.1 System Positioning

diagram

### 3.2 Core Concepts Overview

| Concept | Description | Example |
| --- | --- | --- |
| Provider | Service Provider (Server) | Power Management Module ProviderID=0x01 |
| Consumer | Service Consumer (Client) | A-Core Power Middleware ConsumerID=0x06 |
| Event | Unidirectional Notification (Server→Client) | Power State Notification EventID=1 |
| Method | Request-Response (Client→Server) | Power State Confirmation MethodID=2 |
| Link | Application Layer Connection Handshake | Establish/Disconnect Communication Link |
| Heartbeat | Channel Layer Health Detection | Ping/Pong Every 2 Seconds |
| Stack | Message Stacking for Transmission | 10ms Periodic Batch Transmission |

### 3.3 Communication Channel Configuration

diagram

## 4. Layered Architecture Details

### 4.1 Software Layered Structure

diagram

### 4.2 Module Responsibility Overview

| Layer | Module | Main Responsibility | Key APIs |
| --- | --- | --- | --- |
| Application | pwsm.c | Power state machine logic | Pwsm_Main(), Pwsm_CommEvent() |
| Business Interface | picc_pwr_main.c | Power Event/Method wrapper | Pwr_SendStateNotify(), Pwr_MethodHandler() |
| Service | picc_service.c | Event/Method send & dispatch | PICC_SendEvent(), PICC_RegisterMethodHandler() |
| Stack | picc_stack.c | Message stacking, CRC check | PICC_StackAddMessageToChannel(), PICC_StackProcess() |
| Link | picc_link.c | App-layer connection mgmt | PICC_LinkInit(), PICC_LinkProcess() |
| Heartbeat | picc_heartbeat.c | Channel health detection | PICC_HeartbeatProcess(), PICC_HeartbeatSendPong() |
| Protocol | picc_protocol.c | Packet pack/unpack | PICC_PackMessage(), PICC_UnpackMessage() |

## 5. Core Process Diagrams

### 5.1 System Initialization Process

System initialization uses a high-priority temporary task mode to ensure all modules complete initialization before system operation. The task automatically deletes itself after initialization, releasing resources.

Initialization Steps Description:

- Platform initialization: main() calls Platform_Init() to complete hardware platform initialization

- Create initialization task: Create PICC_Init_Task with priority 4 to ensure priority execution

- Start scheduler: vTaskStartScheduler() starts FreeRTOS scheduling

- Message queue creation: Initialization task first creates the receive message queue g_rxQueue

- IPCF driver initialization: Call ipc_shm_init() to initialize shared memory driver

- Channel initialization: Initialize Channel 1 and Channel 2 separately, Stack and Heartbeat modules for each channel start immediately

- Service layer initialization: PICC_Init() initializes service layer infrastructure

- Link registration: PICC_LinkRegister() registers power management’s application layer connection configuration

- Power module initialization: Pwr_Init() registers Method handler callback functions

- Task self-deletion: Delete self after initialization complete, release stack space

diagram

### 5.2 Message Send/Receive Process

#### 5.2.1 Send Process (M-Core → A-Core)

Message sending uses a stacking buffer + periodic send mechanism to improve transmission efficiency by reducing the number of underlying calls.

Send Process Description:

- Application layer call: Application calls PICC_SendEvent() or PICC_ServiceMethodSend() to initiate send

- Message packing: Service layer calls PICC_PackMessage() to encapsulate data into protocol packet (8-byte header + Payload)

- Add to stacking buffer: Call PICC_StackAddMessageToChannel() to add message to the corresponding channel’s stacking buffer

- Trigger condition check:

  - If buffer not full: Wait for 10ms periodic task to trigger

  - If buffer nearly full: Trigger send immediately to avoid message loss

- Periodic send processing: PICC_StackProcess() executes every 10ms, calls PICC_StackDoSendForChannel()

- Remote ready check: Call ipc_shm_is_remote_ready() to check if A-Core is ready

- Acquire send buffer: Call ipc_shm_acquire_buf() to get IPCF shared memory send buffer

- Fill data: Fill in sequence: CRC_Enable(1B) + protocol data + Counter(2B) + CRC16(2B)

- Low-level send: Call ipc_shm_tx() to send complete stacked packet to shared memory

diagram

#### 5.2.2 Receive Process (A-Core → M-Core)

Message receiving uses an interrupt-triggered + task processing mechanism to ensure real-time performance while avoiding complex operations in ISR.

Receive Process Description:

- Interrupt triggered: After A-Core sends data, IPCF driver triggers data_chan_rx_cb interrupt callback

- Message enqueue: In ISR, only encapsulate message pointer and metadata, push to FreeRTOS queue g_rxQueue, no complex processing

- Task wakeup: App_Rx_Msg_10ms_Task retrieves message from queue (blocking wait)

- Stacking unpack: PICC_StackProcessRx() first verifies CRC16 and Counter

- Layered processing:

  - Heartbeat (Stack layer): Heartbeat packets are directly identified and processed in PICC_StackProcessRx(), not passed to upper layer

  - Link (API layer): Intercepted in PICC_ProcessSingleMessage() and routed to PICC_LinkProcessMessage()

  - Service (Service layer): Other messages enter PICC_ServiceProcessMessage() for dispatch to application callbacks

- Release buffer: After processing, call ipc_shm_release_buf() to release shared memory buffer

diagram

#### 5.2.3 PICC_ServiceProcessMessage Message Dispatch Flow

PICC_ServiceProcessMessage() is the core message dispatch function of the service layer, routing messages to different handler functions based on MessageType.

Message Type to Handler Function Mapping:

[!NOTE] PICC_MSG_LINK_AVAILABLE (0x00) messages are intercepted in the upper layer PICC_ProcessSingleMessage() and routed to PICC_LinkProcessMessage(), they do not enter PICC_ServiceProcessMessage().

| MessageType (Hex) | Message Type | Handler Function | M-Core Role | Description |
| --- | --- | --- | --- | --- |
| 0x05 | REQUEST | PICC_ServiceHandleRequest() | Server | Method request with response |
| 0x06 | REQUEST_NO_RETURN_WITH_ACK | PICC_ServiceHandleRequest() | Server | Request without response but with ACK |
| 0x07 | REQUEST_NO_RETURN_WITHOUT_ACK | PICC_ServiceHandleRequest() | Server | Request without response or ACK |
| 0x08 | NOTIFICATION_WITH_ACK | PICC_ServiceHandleEvent() | Client | Event notification with ACK |
| 0x09 | NOTIFICATION_WITHOUT_ACK | PICC_ServiceHandleEvent() | Client | Event notification without ACK |
| 0x80 | RESPONSE | PICC_ServiceHandleResponse() | Client | Method response (async callback) |
| 0x81 | ACK | Ignored | - | Method middleware confirmation |
| 0x82 | EVENT_ACK | Ignored | - | Event middleware confirmation |

diagram

Detailed Description of Each Processing Branch:

- PICC_LinkProcessMessage (Link Management) - Called in PICC_ProcessSingleMessage

  - Handles connection request, connection response, disconnect notification, disconnect response

  - Maintains Link state machine (DISCONNECTED/CONNECTING/CONNECTED)

- PICC_ServiceHandleRequest (M-Core as Server)

  - Receives Method requests sent by A-Core Client

  - Calls the callback registered by the application layer via PICC_RegisterMethodHandler()

  - Determines reply method based on request type (Response/ACK/no reply)

- PICC_ServiceHandleEvent (M-Core as Client)

  - Receives Event notifications sent by A-Core Server

  - If message type is NOTIFICATION_WITH_ACK, automatically replies EVENT_ACK

  - Calls the callback registered by the application layer via PICC_RegisterEventHandler()

- PICC_ServiceHandleResponse (M-Core as Client)

  - Receives responses from A-Core Server to M-Core Method requests

  - Calls the callback registered by the application layer via PICC_RegisterResponseHandler()

  - Application layer matches original request by SessionID

[!NOTE] ACK type messages (0x81/0x82) are handled internally by middleware, not reported to application layer. Due to real-time requirements, M-Core does not block waiting for ACK return after sending messages with ACK.

### 5.3 Connection State Machine

The Link layer implements application-level connection management. M-Core can act as Client or Server, with different state machine behaviors for each role.

[!NOTE] M-Core’s role is determined by application layer configuration. For example, in the power management module, M-Core is configured as Server (ProviderID=0x01), and A-Core is Client (ConsumerID=0x06).

#### 5.3.1 State Machine When M-Core Acts as Client

When M-Core acts as Client, it needs to actively initiate connection requests to A-Core (Server).

State Transition Trigger Conditions:

- Initialization → CONNECTING: When role == CLIENT is detected in PICC_LinkInit(), it is directly set to CONNECTING state

- CONNECTING → CONNECTED: Received connection confirmation from Server (ReturnCode=0x00)

- CONNECTING → DISCONNECTED: Received rejection response from Server (ReturnCode!=0x00)

- CONNECTED → CONNECTING: After receiving disconnect notification, immediately re-enter CONNECTING state for automatic reconnection

- CONNECTED → CONNECTING: Heartbeat timeout triggers PICC_LinkTriggerReconnect()

- CONNECTING (no response): Continuously send requests at 10ms period with exponential backoff (10→20→40→80→100 cycles)

diagram

Client Mode Key Behaviors:

| State | Behavior Description |
| --- | --- |
| CONNECTING | PICC_LinkProcess() calls PICC_LinkSendMessage() every 10ms to send connection request; exponential backoff enabled on send failure (100ms→200ms→400ms→800ms→1000ms) |
| CONNECTED | Can send Method requests, listen for Server’s Event notifications |
| DISCONNECTED | Wait for application layer to trigger reconnection (current code enters this state when Server rejects) |

[!NOTE] Exponential Backoff Mechanism: When send buffer is full or send fails, PICC_LinkProcess() uses exponential backoff to avoid frequent retries. Sequence: 10→20→40→80→100 cycles (i.e., 100ms→200ms→400ms→800ms→1000ms).

#### 5.3.2 State Machine When M-Core Acts as Server

When M-Core acts as Server, it is in a passive waiting state, waiting for A-Core (Client) to initiate connection requests.

State Transition Trigger Conditions:

- Initialization → DISCONNECTED: When role == SERVER is detected in PICC_LinkInit(), set to DISCONNECTED state

- DISCONNECTED → CONNECTED: Received connection request from Client, enter CONNECTED after sending confirmation

- CONNECTED → DISCONNECTED: Received disconnect notification or Heartbeat timeout

diagram

Server Mode Key Behaviors:

| State | Behavior Description |
| --- | --- |
| DISCONNECTED | Passively wait for Client to initiate connection request, PICC_LinkHandleRequest() handles received requests |
| CONNECTED | Can send Event notifications, handle Client’s Method requests |

[!IMPORTANT] In Server mode there is no CONNECTING state. After receiving Client request, PICC_LinkHandleRequest() sends confirmation response and directly enters CONNECTED state.

#### 5.3.3 Link Message Subtypes

| Subtype | Enum Value | Description |
| --- | --- | --- |
| PICC_LINK_CONNECT | 0x01 | Connection request/response |
| PICC_LINK_DISCONNECT | 0x02 | Disconnect notification/response |
| PICC_LINK_RECONNECT | 0x03 | A-Core Server restart notification, triggers Client to reconnect |

### 5.4 Heartbeat Detection Mechanism

The Heartbeat layer implements channel-level health detection, running independently of Link connection state. Heartbeat is always working regardless of whether the application layer is connected.

Heartbeat Mechanism Key Parameters:

| Parameter | Value | Description |
| --- | --- | --- |
| Period | 2000ms | Send Ping every 2 seconds |
| Timeout Threshold | 3 times | Fault determined after 3 consecutive missed Pongs |
| Packet Format | 9 bytes | Fixed format, does not follow standard protocol header |

Heartbeat Workflow:

- Normal case: M-Core sends Ping every 2 seconds, A-Core immediately replies Pong upon receipt, M-Core resets miss_count after receiving Pong

- Anomaly detection: If Pong is not received after Ping is sent, miss_count increments

- Fault determination: When miss_count >= 3, channel fault is determined

- Fault handling:

  - Call PICC_LinkTriggerReconnect() to set Link state to DISCONNECTED

  - Notify application layer via callback

Key Notes: 1. M-Core sends Ping: Actively sends every 2 seconds, expecting Pong from A-Core 2. M-Core receives Ping: When A-Core sends Ping, M-Core immediately replies Pong (does not affect its own missCount) 3. M-Core receives Pong: Received A-Core’s reply to its own Ping, resets missCount=0 4. Independent counting: M-Core and A-Core each maintain their own missCount, not affecting each other 5. Timeout trigger: After 3 consecutive misses (6 seconds), clear buffer and reconnect —

## 6. Module Detailed Design

### 6.1 Protocol Packet Format

#### 6.1.1 Protocol Header Structure (8 bytes)

diagram

#### 6.1.2 Stacked Packet Format

diagram

CRC Enable: 0x00 = Enable, 0x01 = Disable

#### 6.1.3 Message Type Enumeration

| Value (Hex) | Name | Direction | Description |
| --- | --- | --- | --- |
| 0x00 | LINK_AVAILABLE | Bidirectional | Connection/disconnect handshake |
| 0x05 | REQUEST | Client→Server | Method request with response |
| 0x06 | REQUEST_NO_RETURN_WITH_ACK | Client→Server | No response but with ACK |
| 0x07 | REQUEST_NO_RETURN_WITHOUT_ACK | Client→Server | No response no ACK |
| 0x08 | NOTIFICATION_WITH_ACK | Server→Client | Event with ACK |
| 0x09 | NOTIFICATION_WITHOUT_ACK | Server→Client | Event without ACK |
| 0x80 | RESPONSE | Server→Client | Method response |
| 0x81 | ACK | - | Method confirmation |
| 0x82 | EVENT_ACK | Client→Server | Event confirmation |

### 6.2 CRC16 Algorithm

/* Algorithm parameters (must be consistent with A-Core) */
#define xorValueCrc16   0x0000U
#define startValueCrc16 0xFFFFU

/* Lookup table in picc_protocol.c */
uint16 PICC_CRC16(const uint8 *data, uint32 len);

### 6.3 Key Data Structures

#### 6.3.1 PICC Initialization Configuration

typedef struct {
    uint8        linkLocalId;     /* Local ID (Client=ConsumerID, Server=ProviderID) */
    uint8        linkRemoteId;    /* Remote ID (Client=ProviderID, Server=ConsumerID) */
    PICC_Role_e  linkRole;        /* Role: PICC_ROLE_CLIENT / PICC_ROLE_SERVER */
    uint8        channelId;       /* IPCF Channel ID */
} PICC_InitConfig_t;

#### 6.3.2 Link Context

typedef struct {
    PICC_LinkConfig_t config;           /* Link configuration */
    volatile PICC_LinkState_e state;    /* Current state */
    boolean           isInitialized;    /* Whether initialized */
} PICC_LinkContext_t;

### 6.4 Asynchronous Send and Request Mechanism

Due to M-Core’s real-time requirements, synchronous blocking wait is not supported. All send operations are asynchronous, with results obtained via callbacks or flag polling.

#### 6.4.1 Asynchronous Send Implementation Principle

M-Core’s asynchronous sending is based on message stacking + periodic flush mechanism:

diagram

Key Function Descriptions:

| Function | File | Functionality | Return Value |
| --- | --- | --- | --- |
| PICC_SendEvent() | picc_service.c | Send Event notification | 0=success, non-0=fail |
| PICC_ServiceMethodSend() | picc_service.c | Send Method request | SessionID (>0=success) |
| PICC_StackAddMessageToChannel() | picc_stack.c | Add message to stack buffer | 0=success |
| PICC_StackProcess() | picc_stack.c | Periodic buffer flush | None |

#### 6.4.2 Async Method Request and Response Matching

When M-Core acts as Client to send Method requests, it needs to match requests and responses via SessionID.

Request Flow:

- Call PICC_ServiceMethodSend() to send request

- Function returns auto-allocated SessionID (1~255 cycling)

- Application saves this SessionID for later matching

Response Receive Flow:

- Register response callback: PICC_RegisterResponseHandler(callback)

- When Response is received, middleware calls the callback function

- In the callback, match original request by sessionId parameter

/* Response callback function prototype */
typedef void (*PICC_ResponseCallback_t)(
    uint8 providerId,   /* Service Provider ID */
    uint8 methodId,     /* Method ID */
    uint8 sessionId,    /* Session ID for request matching */
    uint8 returnCode,   /* Return code */
    const uint8 *rspData, /* Response data */
    uint16 rspLen       /* Response data length */
);

#### 6.4.3 Asynchronous Receive Processing Mechanism

M-Core uses flag polling mode to receive messages:

diagram

Processing Stages: | Stage | Context | Function | Description | |——-|———|———-|————-| | 1 | ISR | data_chan_rx_cb | Enqueue message to FreeRTOS queue | | 2 | Rx Task | PICC_ServiceProcessMessage | Dispatch and call callback | | 3 | App Task | Pwsm_Main | Poll flag every 10ms |

Why not process business logic directly in callback:

- Callback executes in Rx task context with higher priority

- Complex state machine logic may cause unpredictable task execution time

- Flag decoupling ensures state machine executes in fixed 10ms cycle

### 6.5 Debug Diagnostic Variables

To facilitate problem analysis using TRACE32 or other debug tools, the system provides multiple global diagnostic variables.

#### 6.5.1 Communication Trace Variables (g_piccTrace)

Location: picc_trace.c / picc_trace.h

Purpose: Record recent TX/RX messages for analyzing communication content.

extern PICC_ChannelTrace_t g_piccTrace;

| Member | Type | Description |
| --- | --- | --- |
| g_piccTrace.tx.data[row][0] | uint8 | Send channel ID |
| g_piccTrace.tx.data[row][1] | uint8 | Send data length |
| g_piccTrace.tx.data[row][2..] | uint8[] | Send data content |
| g_piccTrace.tx.writeIndex | uint8 | Current write row index (0~9 cycling) |
| g_piccTrace.rx.data[row][0] | uint8 | Receive channel ID |
| g_piccTrace.rx.data[row][1] | uint8 | Receive data length |
| g_piccTrace.rx.data[row][2..] | uint8[] | Receive data content |
| g_piccTrace.rx.writeIndex | uint8 | Current write row index (0~9 cycling) |

Usage: Observe g_piccTrace.tx.data and g_piccTrace.rx.data arrays in TRACE32.

#### 6.5.2 Power Module Diagnostic Variables (g_pwrDiag / g_pwsmDiag)

Location: picc_pwr_main.c

Purpose: Track key events and counts during power shutdown process.

| Variable | Description |
| --- | --- |
| g_pwrDiag.methodHandlerCallCount | Pwr_MethodHandler() total call count |
| g_pwrDiag.doneHandlerCallCount | Method ID=8 process count |
| g_pwrDiag.stateAckHandlerCallCount | Method ID=2 process count |
| g_pwrDiag.ctrlAckHandlerCallCount | Method ID=11 process count |
| g_pwrDiag.phase1DonePayloadByte0 | Last ID=8 payload[0] |
| g_pwrDiag.phase1DoneFlagSetCount | Flag set count |
| g_pwrDiag.lastMethodId | Last processed MethodID |
| g_pwrDiag.lastSendCtrlCmdResult | Pwr_SendCtrlCmd() return value |
| g_pwsmDiag.commEventCallCount | Pwsm_CommEvent() call count |
| g_pwsmDiag.timeoutCounterAtFlagSet | Timeout counter value at flag set |
| g_pwsmDiag.transitionToTxId4 | Whether transitioned to send ID=4 state |
| g_pwsmDiag.transitionToShutdownDone | Whether transitioned to shutdown done state |

#### 6.5.3 Link/Stack State Variables

Location: picc_link.c / picc_stack.c

| Variable | Description | Observation Method |
| --- | --- | --- |
| g_linkContexts[i].state | Link connection state | View PICC_LinkState_e enum value |
| g_linkContexts[i].isInitialized | Whether initialized | TRUE/FALSE |
| g_linkContexts[i].config.role | Role (Client/Server) | View PICC_Role_e enum value |
| g_stackInstances[i].context.usedSize | Current pending send buffer length | Numeric value |
| g_stackInstances[i].context.txCounter | Send counter | Cumulative value |
| g_stackInstances[i].context.rxCounter | Receive counter | Cumulative value |

#### 6.5.4 Application Layer Statistics Variables (App_Data_t)

Location: main.c

| Variable | Description |
| --- | --- |
| g_appData.rx_count | Receive message count |
| g_appData.tx_count | Send/process message count |
| g_appData.error_count | Error count |
| g_appData.last_error | Last error code |
| g_appData.link_state | Link connection state |
| g_appData.last_rx_ch | Last receive channel |

## 7. Task Scheduling Design

### 7.1 FreeRTOS Task List

| Task Name | Priority | Stack Size (words) | Stack Size (Bytes) | Function | Lifecycle |
| --- | --- | --- | --- | --- | --- |
| PICC_Init_Task | 4 (Highest) | 256 | 1024 (1KB) | One-time init | Self-delete after init |
| App_Rx_Msg_10ms_Task | 3 | 192 | 768 | Message RX process | Permanent |
| task_M7_0_10ms | 2 | 256 | 1024 (1KB) | Periodic task | Permanent |

### 7.2 10ms Periodic Task Content

void task_M7_0_10ms(void *pvParameters)
{
    for(;;) {
        /* 1. Stack layer: Send stacked messages in buffer */
        PICC_StackProcess();

        /* 2. Heartbeat layer: Send Ping every 2000ms, check timeout */
        PICC_HeartbeatProcess();

        /* 3. Link layer: Send connection request every 10ms in Client mode */
        PICC_LinkProcess();

        /* 4. Power state machine */
        Pwsm_Main();

        vTaskDelayUntil(&xLastWakeTime, pdMS_TO_TICKS(10));
    }
}

## Appendix A: Packet Examples

### A.1 Connection/Disconnection Packet Examples

| Step | Direction | Description | Protocol Header (Hex) | Payload (Hex) |
| --- | --- | --- | --- | --- |
| 1 | Client→Server | Connection request | 01 00 06 0000 01 00 05 | 01 (Connect) |
| 2 | Server→Client | Connection confirm | 01 00 06 0000 00 00 05 | 01 (Connect) |
| 3 | Server→Client | Disconnect notify | 01 00 06 0000 01 00 01 | 02 (Disconnect) |
| 4 | Client→Server | Disconnect confirm | 01 00 06 0000 00 00 01 | 02 (Disconnect) |

[!NOTE]

Power Management Example: M-Core is SERVER, A-Core is Client. The following uses ProviderID=0x01, ConsumerID=0x06 as example values.

- ProviderID=0x01, ConsumerID=0x06

- MessageType=0x00 (LINK_AVAILABLE)

- ReturnCode: 0x01=Disagree/Requesting, 0x00=Agree

- Payload[0]: 0x01=Connect packet, 0x02=Disconnect packet

- Length: Connect=0x0005, Disconnect=0x0001

### A.2 Heartbeat Packet Examples

| Step | Direction | Description | Complete Packet (Hex) |
| --- | --- | --- | --- |
| 1 | M→A | Ping | FF 00 FF 00 FF 00 00 01 00 |
| 2 | A→M | Pong | FF 00 FF 00 FF 00 00 01 01 |

[!NOTE] Heartbeat packets are fixed at 9 bytes and do not follow the standard protocol header. Last byte: 0x00=Ping, 0x01=Pong

## Appendix B: Design Constraints and Notes

[!IMPORTANT] M-Core Real-time Constraint: M-Core does not support synchronous waiting, all operations must be asynchronous.

[!WARNING] Heartbeat Independence: Heartbeat starts immediately after channel initialization, independent of Link connection state.

[!CAUTION] Counter Non-reset: M-Core’s send counter (Stack layer) must not be reset to zero after disconnect and reconnect.

[!NOTE] SessionID Range: 0x01~0xFF, rolls back to 0x01 after reaching 0xFF, only used for Event with ACK and async Method.

End of Document